# SEA SENTRY (PHASE-3)

## (FISHERMAN BORDER ALERT SYSTEM)

TEAM MEMBERS:

AASHIKA M

INIYA M

DEEPIKA  G

SYSTEM ARCHITECTURE:

OVERALL SYSTEM ARCHITECTURE:

- NEO-6M GPS Module:

- The NEO-6M GPS module is used to obtain the geographical position of the fisherman. It provides important location information such as latitude and longitude.

- Arduino Mega 2560:

The Arduino Mega 2560 acts as the central processing unit of the system. It receives the location data and processes it to determine the fisherman’s position with respect to the predefined maritime boundary.

- Boundary Monitoring:

The obtained GPS coordinates are compared with the predefined safe-zone and boundary coordinates. This enables the system to identify whether the fisherman is within the safe region or approaching/crossing the boundary.

- Warning Generation:

When the fisherman approaches a critical boundary region, the Arduino generates a warning signal. This provides an opportunity for the fisherman to take corrective action.

- 3-in-1 LED Indicator:

The three-color LED provides a visual indication of different system conditions. Different colors can represent safe, warning, and danger/boundary-crossing conditions.

- Buzzer Alert:

The buzzer provides an audible warning when the fisherman reaches a warning or danger condition. Different buzzer patterns can be used for different levels of alerts.

- PROTEUS Simulation:

PROTEUS is used to simulate and test the system before implementing the physical prototype. It helps verify the Arduino program, circuit connections, input conditions, and alert responses virtually.

- Real-Time Monitoring:

The system is designed to continuously process the location information so that changes in the fisherman’s position can be monitored during operation.

- Early Warning:

The main purpose of the architecture is to provide an early warning before the fisherman crosses the predefined boundary, helping improve awareness and safety.

- Overall Working:

Thus, the system follows the basic flow:

GPS Location → Arduino Processing → Boundary Checking → Warning Decision → LED + Buzzer Alert.

DATA FLOW DIAGRAM:

FUNCTIONAL BLOCK DIAGRAM:

The functional block diagram of the Sea Sentry system represents the major functions performed by each component and the flow of information between them. The system begins with the NEO-6M GPS module, which receives signals from GPS satellites and provides the current latitude and longitude of the fisherman.

The GPS coordinates are then given to the Arduino Mega 2560, which acts as the main processing unit. The Arduino processes the received location information and compares it with the predefined maritime boundary coordinates. Based on this comparison, the system determines the current status of the fisherman, such as safe, warning, or danger.

According to the detected status, the Arduino controls the 3-in-1 LED indicator to provide a visual indication. At the same time, the buzzer generates an audible alert, with different alert patterns representing different warning conditions.

The complete functionality is simulated and tested using PROTEUS, which allows the circuit operation and program logic to be verified before implementing the physical prototype.

HARDWARE SOFTWARE PARTITIONING:

Hardware part:

- Arduino Mega 2560 – main processing/controller hardware

- NEO-6M GPS module – obtains location coordinates

- 3-in-1 LED – visual warning indicator

- Buzzer – audible warning

- Connecting wires and power supply – provide electrical connections and power

Software part:

- The Arduino program contains the logic for reading GPS data.

- It processes the latitude and longitude values.

- It compares the current position with the predefined boundary coordinates.

- It determines the system status, such as safe, warning, or danger.

- It controls the LED and buzzer according to the detected status.

- PROTEUS is used to simulate and test the hardware and software

HARDWARE DESIGN:

FINAL BLOCK DIAGRAM:

CIRCUIT SCHEMATICS:

COMPONENT SELECTION:

## 1. Processing Core: Arduino Mega 2560

- Multi-UART Architecture: Features 4 independent hardware serial ports. This allows dedicated communication with the GPS module on Serial1 [1] without interrupting debugging or locking up the processor loop.

- Memory Buffer: Packed with 8 KB of SRAM, providing ample memory to process and parse long incoming GPS NMEA text strings without crashing.

## 2. Telemetry Unit: Neo-6M GPS Module

- Autonomous Operation: Tracks global satellite coordinates independently without relying on Wi-Fi or cellular networks.

- Standardized Output: Converts complex satellite signals into standard NMEA-0183 ASCII data sentences, transmitting clean location updates to the Arduino Mega using minimal power (approx. 45mA).

## 3. Alert Interface: 3-Tier LED Array & Active Buzzer

- Visual Recognition: Uses distinct high-efficiency Green, Yellow, and Red LEDs for instant status awareness (Safe, Searching, or Geofence Breach) under open daylight conditions.

- Acoustic Penetration: Uses a single Active Buzzer driven at three distinct pulse frequencies to pierce through outdoor marine noise (wind, waves, engines) matching the risk levels.

## 4. System Protection: 220 Ω Resistors & DC Battery

- Current Regulation: Incorporates 220 Ω inline resistors to limit current draw to roughly 13.6mA, protecting the Arduino pins and LEDs from electrical burnout.

- Regulated Power: A standalone 9V–12V DC Battery offers mobile, stable power, relying on the Arduino’s onboard voltage regulator to down-step it cleanly to a safe 5V rail.

BILL OF MATERIALS (BOM):

SOFTWARE DESIGN:

FLOWCHARTS:

ALGORITHMS:

- Step 1: Define the target safe coordinates (SAFE_LAT, SAFE_LON) and the maximum allowable boundary radius (SAFE_RADIUS_METERS).

- Step 2: Configure the data direction registers by setting Pin 13 (Green LED), Pin 12 (Yellow LED), and Pin 11 (Red LED) as OUTPUT.

- Step 3: Set Pin 10 (Active Buzzer) as OUTPUT to drive the audio alerts.

- Step 4: Open the primary debugging hardware port (Serial0) at 9600 baud to stream text directly to your Proteus Virtual Terminal window.

- Step 5: Open the auxiliary serial hardware port (Serial1) at 9600 baud to establish a direct UART data line with the Neo-6M GPS Module.

- Step 6: Read the incoming data buffer on Serial1 to check for active NMEA-0183 sentences from satellites.

- Step 7: IF there is no valid data stream or satellite lock, turn on the Yellow LED (Pin 12), pulse a slow warning chirp on the Buzzer (Pin 10), and loop back to Step 6.

- Step 8: ELSE (when satellite lock is successful), extract the current latitude and longitude variables from the incoming $GPRMC text string.

- Step 9: Calculate the real-time distance between your current coordinates and the safe target coordinates using the Haversine mathematical equation.

- Step 10: IF the calculated distance is less than or equal to SAFE_RADIUS_METERS, turn on the Green LED (Pin 13) and turn off all other LEDs and the buzzer.

- Step 11: ELSE (if the vessel drifts outside the boundary), turn on the Red LED (Pin 11) and drive a continuous, high-frequency emergency siren on the Buzzer (Pin 10).

- Step 12: Print the live distance and alarm status variables to the Proteus Virtual Terminal, delay the system for 500 milliseconds, and loop back to Step 6.

COMMUNICATION PROTOCOLS:

- Protocol Type: Universal Asynchronous Receiver-Transmitter (UART) / Serial Communication.

- Data Layout Configuration: 8 Data Bits, No Parity, 1 Stop Bit (Standard 8N1 framing).

- Baud Rate Speed: Fixed at 9600 bps (bits per second) to match the native hardware output of the Neo-6M GPS module.

- Physical Interconnect Layer: Standard TTL logic levels (0V for Logic LOW, 5V for Logic HIGH).

## 2. Dual-Channel Protocol Architecture

The Arduino Mega 2560 acts as the communication hub, running two separate hardware UART channels simultaneously without data collisions:

## Channel 1: Telemetry Ingest (Serial1)

- Source Device: Neo-6M GPS Module (Pin: TX)

- Destination Device: Arduino Mega 2560 (Pin 19: RX1)

- Data Flow: Unidirectional (Receive Only). The GPS continuously pushes geolocation data packs into the Mega's serial hardware ring buffer.

## Channel 0: Diagnostic Telemetry (Serial0)

- Source Device: Arduino Mega 2560 (Pin 1: TX0)

- Destination Device: Proteus Virtual Terminal Instrument (Pin: RXD)

- Data Flow: Unidirectional (Transmit Only). The Mega streams human-readable distance status metrics directly to the simulation screen monitor.

## 3. Application Data Layer (NMEA-0183 Payload)

The data payload passing over the UART wire follows the global NMEA-0183 marine electronics text standard. The system captures and processes the $GPRMC (Recommended Minimum Specific GNSS Data) sentence packet frame structure:

- String Frame Synchronization: The hardware looks for the ASCII $ symbol to trigger the parsing loop.

- Validation Payload Field: The third array block indicates lock quality (A = Valid Satellite Data, V = Void Data/Searching).

- Payload Extraction Matrix: When the state reads A, the software parses the target latitude and longitude fields directly out of the comma-separated ASCII sequence to calculate your geofence boundary boundaries.

SIMULATION: