# Automatic Railway Gate Control System Using IR Sensor

Arduino Program (Servo + 2 IR Sensors)

#include <Servo.h>

Servo gateServo;

const int IR1_PIN = 2;

const int IR2_PIN = 3;

const int SERVO_PIN = 9;

const int GATE_OPEN = 90;

const int GATE_CLOSED = 0;

// 0 = Waiting for train

// 1 = Train detected by Sensor 1

// 2 = Train detected by Sensor 2

int state = 0;

bool confirmedDetect(int pin) {

for (int i = 0; i < 5; i++) {

if (digitalRead(pin) != LOW) return false;  // any HIGH = noise, reject

delay(10);

}

return true;  // stayed LOW ~50ms straight = real object

}

void setup()

{

Serial.begin(9600);

pinMode(IR1_PIN, INPUT);

pinMode(IR2_PIN, INPUT);

gateServo.attach(SERVO_PIN);

// Initially gate OPEN

gateServo.write(GATE_OPEN);

Serial.println("Railway Gate System Started");

Serial.println("Gate OPEN");

}

void loop()

{

int S1 = digitalRead(IR1_PIN);

int S2 = digitalRead(IR2_PIN);

// -----------------------------

// STATE 0: Waiting for train

// -----------------------------

if (state == 0)

{

if (confirmedDetect(IR1_PIN))

{

gateServo.write(GATE_CLOSED);

Serial.println("Sensor 1 Detected -> Gate CLOSED");

state = 1;

}

}

// -----------------------------

// STATE 1: Train has crossed S1

// -----------------------------

else if (state == 1)

{

// IMPORTANT:

// Even if S1 becomes HIGH,

// gate MUST remain CLOSED.

gateServo.write(GATE_CLOSED);

if (confirmedDetect(IR2_PIN))

{

Serial.println("Sensor 2 Detected -> Train Passing");

state = 2;

}

}

// -----------------------------

// STATE 2: Train detected by S2

// -----------------------------

else if (state == 2)

{

// Keep gate CLOSED

gateServo.write(GATE_CLOSED);

// S2 becomes HIGH = train has crossed S2

if (S2 == HIGH)

{

Serial.println("Train Cleared -> Gate OPEN");

gateServo.write(GATE_OPEN);

state = 0;

}

}

delay(100);

}