# AUTOMATIC RAILWAY GATE CONTROL SYSTEM USING IR SENSOR

## Phase 2 — Ideation & Concept Development

Submitted by, M HARINI, SM AKSHAYA, P THARANI LAKSHMI

# 1. Brainstorming Report

## 1.1 Detection Ideas

- Dual IR sensor pairs (approach + departure) as the core detection method — carried forward from Phase 1.

- Debounce/validation logic that requires the IR beam to stay broken for a minimum duration before triggering, to reject birds, debris, or momentary interference.

- Redundant sensor pair on each side (2 IR pairs per side) as a stretch goal for fail-safe detection.

- Optional IR receiver signal-strength logging to help tune sensor placement distance during testing.

## 1.2 Gate Actuation Ideas

- Servo-motor-driven barrier arm for the prototype scale (low torque, easy to control via microcontroller PWM).

- Geared DC motor with limit switches as an alternative for a sturdier barrier in later scale-up.

- Fail-safe default: gate defaults to closed if the controller loses power or sensor signal, rather than defaulting open.

## 1.3 Alerting Ideas

- LED + buzzer activated the moment the approach sensor triggers, before the gate starts moving, to give road users advance warning.

- Distinct buzzer pattern (intermittent vs continuous) to differentiate 'gate closing' from 'gate closed / train in zone'.

- Optional signage-style blinking LED strip along the barrier arm for night-time visibility.

## 1.4 Documentation & Process Ideas

- Continue the Phase 1 Git/GitHub workflow, adding a firmware branch and hardware bring-up checklist.

- Log every detection/gate event to serial output during testing to build an evidence trail for the validation report.

# 2. Idea Prioritization Matrix

| Idea | Impact (1-5) | Feasibility (1-5) | Priority |
| --- | --- | --- | --- |
| Dual IR sensor detection (approach + departure) | 5 | 5 | Now |
| Debounce / validation logic against false triggers | 5 | 4 | Now |
| Fail-safe default-closed behavior | 5 | 5 | Now |
| LED + buzzer early warning | 4 | 5 | Now |
| Servo-driven barrier arm | 4 | 4 | Now |
| Distinct buzzer patterns for closing vs closed | 3 | 4 | Next |
| Serial event logging for test validation | 3 | 5 | Next |
| Redundant sensor pair per side | 3 | 2 | Later |
| Geared DC motor + limit switches | 3 | 2 | Later |
| Blinking LED strip on barrier arm | 2 | 3 | Later |

"Now" items form the base implementation carried into the Functional Requirements below. "Next" items are planned once the base system is validated. "Later" items remain optional future-work extensions, consistent with the scope boundary set in Phase 1.

# 3. Empathy Map

Primary stakeholder: a daily commuter (motorist or pedestrian) crossing an unmanned rural level crossing.

| Says | Thinks |
| --- | --- |
| "I never know if a train is actually coming or if the gate is just stuck." / "There's no gatekeeper here after dark." | "Is it safe to cross right now?" / "I hope the warning gives me enough time to stop." |

| Does | Feels |
| --- | --- |
| Slows down and looks/listens manually at the crossing; sometimes crosses without waiting for a formal signal when no gate is visibly closed. | Anxious and uncertain near unmanned crossings; frustrated by inconsistent or delayed gate operation. |

| Pains | Gains |
| --- | --- |
| Ambiguous or delayed signals; risk of misjudging an approaching train; no consistent warning at unmanned crossings. | A clear, early, and consistent visual + audible warning; a gate that closes reliably before the train arrives and reopens promptly after. |

# 4. User Persona

| Attribute | Details |
| --- | --- |
| Name | Ramesh Kumar |
| Role | Daily two-wheeler commuter, rural route |
| Context | Crosses an unmanned level crossing twice a day on the way to work; crossing has no dedicated gatekeeper |
| Goals | Cross safely and quickly; trust that a closed gate always means a train is genuinely present |
| Frustrations | Manually-operated gates near his route are sometimes left open due to gatekeeper absence or scheduling errors |
| Needs from the system | Early, unambiguous LED/buzzer warning; gate that closes only when a train is actually detected (no false closures that waste his time); gate that reopens promptly once the train clears |

A secondary persona — the maintenance engineer/faculty mentor responsible for validating and servicing the prototype — is addressed indirectly through the documentation, logging, and Git/GitHub workflow requirements rather than a separate empathy map, since their needs are process-oriented rather than experiential.

# 5. Proposed Solution

Based on the brainstorming, prioritization, and stakeholder needs above, the proposed solution refines the Phase 1 concept into a concrete system design:

- A dual IR sensor pair is installed on the approach side and a second pair on the departure side of the crossing, at a distance calculated from the expected train speed range.

- A microcontroller (Arduino-family, per Phase 1 scope) continuously polls both sensor pairs and applies a debounce/validation routine before accepting a detection as genuine.

- On a validated approach-side trigger, the controller immediately activates the LED and buzzer, then drives the servo-actuated barrier to the closed position.

- The gate remains closed and the alert remains active for as long as the train occupies the crossing zone, as confirmed by the sensor states.

- On a validated departure-side trigger (train clear of the crossing), the controller deactivates the alert and returns the barrier to the open position, resetting for the next cycle.

- If power or sensor signal is lost, the system defaults to the closed/alerted state rather than open, prioritizing road-user and rail safety over convenience.

This solution directly targets the gaps identified in the Phase 1 gap analysis: it uses dual-sensor redundancy instead of a single point of failure, parameterized rather than fixed timing, and an explicit debounce routine — while staying within the low-cost, non-networked scope the literature review found underserved.

# 6. Functional Requirements

- The system shall detect an approaching train using the approach-side IR sensor pair.

- The system shall validate a detection using debounce logic before triggering any action, to reject momentary  interruptions.

- The system shall activate the LED and buzzer immediately upon a validated approach-side detection, before gate movement begins.

- The system shall close the gate barrier automatically once a train approach is validated.

- The system shall keep the gate closed and the alert active for the entire duration the train occupies the crossing zone.

- The system shall detect train departure using the departure-side IR sensor pair.

- The system shall reopen the gate and deactivate the alert automatically once departure is validated.

- The system shall reset to a ready state after each full detection cycle, able to respond to the next train without manual reset.

- The system shall log detection and gate-state events (e.g., via serial output) to support testing and validation.

# 7. Non-functional Requirements

- Reliability: the false-trigger rate under laboratory test conditions (debris/ambient IR interference) shall be minimized through debounce validation, targeting no false gate closures during standard test runs.

- Response time: the LED/buzzer alert shall activate within a small, consistent delay of a validated detection, well ahead of gate closure, to maximize road-user warning time.

- Safety: the default state on power loss or sensor fault shall be closed/alerted, never open, consistent with the fail-safe principle in the proposed solution.

- Low cost: total component cost shall remain within a student-prototype budget, avoiding hybrid IR+inductive/RFID or IoT/GSM additions per the Phase 1 scope boundary.

- Maintainability: firmware and circuit documentation shall be version-controlled on GitHub with clear commit conventions, enabling any team member or mentor to pick up development.

- Scalability: the sensor-to-gate timing shall be parameterized (not hard-coded) so the design can later be tuned for different track speeds or crossing widths.

- Usability: alert signals (LED/buzzer) shall be  distinguishable and intuitive to road users without prior training.

# 8. Project Planning (Timeline)

The table below presents an 8-week Phase 2 timeline in Gantt-chart form, covering ideation through to Phase 2 submission.

| Task | W1 | W2 | W3 | W4 | W5 | W6 | W7 | W8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Requirement finalization & empathy research |  |  |  |  |  |  |  |  |
| Brainstorming & idea prioritization |  |  |  |  |  |  |  |  |
| Persona & proposed solution design |  |  |  |  |  |  |  |  |
| Functional/non-functional requirement spec |  |  |  |  |  |  |  |  |
| Circuit design & component procurement |  |  |  |  |  |  |  |  |
| Firmware development (sensor + control logic) |  |  |  |  |  |  |  |  |
| Gate actuation & alert integration |  |  |  |  |  |  |  |  |
| Testing, debounce tuning & validation |  |  |  |  |  |  |  |  |
| Risk mitigation & documentation |  |  |  |  |  |  |  |  |
| Phase 2 review & submission |  |  |  |  |  |  |  |  |

# 9. Risk Analysis

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| False gate triggering from birds, debris, or ambient IR | Medium | High | Debounce/validation logic requiring sustained beam interruption before acting; tune threshold during testing |
| Sensor misalignment during installation | Medium | Medium | Fixed mounting jigs and a documented alignment/calibration checklist |
| Gate fails to reopen after train departure (stuck actuator) | Low | High | Servo/motor limit checks with a timeout fallback and manual-override switch for testing |
| Power loss during operation | Low | High | Fail-safe default-closed behavior; consider backup power for future field deployment |
| Timing mismatch for varying train speeds | Medium | Medium | Parameterized sensor-to-gate distance/timing rather than a fixed constant, per Functional Requirements |
| Component procurement delays | Medium | Medium | Order long-lead components (servo, sensors) in Week 1; identify substitute part numbers in advance |
| Scope creep toward RFID/GSM/IoT extensions | Medium | Low | Extensions explicitly deferred to future work per Phase 1 scope; tracked separately in GitHub issues |
| Team coordination/version-control conflicts | Low | Medium | Continue Phase 1 Git/GitHub branching and pull-request workflow with regular reviews |